Baofeng UV-5RM Pro GPS - Protocol and Connection Parameters

I. Connection Parameters

Both the USB and Serial interfaces utilize the same base COM port configurations.

| Parameter | Value |
| --- | --- |
| Baud Rate | 19200 |
| Data Bits | 8 |
| Stop Bits | 1 |
| Parity | None |
| Flow Control | Hardware (RTS/DTR control manipulated during setup, with SERIAL_RTS_CONTROL active) |
| XON / XOFF Limits | Initialized at 17206 / 17206 |
| Wait Mask | SERIAL_EV_RXCHAR |

II. Protocol Encoding Discovery (XOR Cipher)

By comparing the two captures, there is a crucial structural discovery regarding how the data is encoded on the wire: One of the captures utilizes a strict 0xA5 XOR cipher for all data following the initial wake-up sequence.

- Cleartext Sequence: The first capture sends the authentication payload in cleartext (e.g., 6d 6f 72 7a 6f 7c 70 3d, which is ASCII morzo|p=).

- Ciphertext Sequence: The second capture sends the exact same sequence, but every byte is XOR'd with 0xA5 (e.g., c8 ca d7 df ca d9 d5 98).

  - Proof: 0x6D ^ 0xA5 = 0xC8 | 0x6F ^ 0xA5 = 0xCA.

- Exception: The initial 16-byte wake-up sequence (00 and FF blocks) and the device's first 0x41 ('A') response are strictly exempt from the cipher and are transmitted raw in both captures.

Note: The analysis below is documented using the "cleartext" base values from the first capture for readability.

III. Communication Sequence

The communication is strictly synchronous (Request -> Response) and operates in three distinct phases:

Phase 1: Wake-up / Synchronization

- Host TX: 00 00 00 00 00 00 00 00 00 00 00 00 FF FF FF FF (12 null bytes followed by 4 0xFF bytes)

- Device RX: 41 (ASCII A)

Phase 2: Authentication & Initialization

- Host TX: 6D 6F 72 7A 6F 7C 70 3D (ASCII morzo|p=)

- Device RX: 7C (ASCII |)

- Host TX: C2 C2 C2 C2 C2 C2 C2 C2

- Device RX: 7C (ASCII |)

- Host TX: 74 73 7B 72 6F 70 7C 69 74 72 73 (ASCII ts{rop|itrs)

- Device RX: 7F 7B 10 08 6F 75 C2 C2 3D C2 C2 C2 C2 C2 C2 C2 (16-byte payload)

- Host TX: 6F (ASCII o)

- Device RX: 7C (ASCII |)

Phase 3: Data Read Loop

The host triggers sequential block reads using a 4-byte command. The command structure consists of an operation byte (0x6F / o), a block index byte, and trailing padding (0x3D 0x3D / ==).

The device responds with a large block of data that echoes the requested index in its header (0x6A / j, followed by the index, then ==...).

- Block Request 1:

  - Host TX: 6F 3D 3D 3D (o===)

  - Device RX: Large payload starting with header 6A 3D 3D 3D (j===)

- Block Request 2:

  - Host TX: 6F 2D 3D 3D (o-==)

  - Device RX: Large payload starting with header 6A 2D 3D 3D (j-==)

- Block Request 3:

  - Host TX: 6F 1D 3D 3D (o\x1D==)

  - Device RX: Large payload starting with header 6A 1D 3D 3D (j\x1D==)

Loop Behavior: The index byte in the read command decrements by 0x10 sequentially for each iteration. The index sequence observed is:

- 0x3D

- 0x2D

- 0x1D

- 0x0D

- 0xFD (Rolls over to F after 0)

- 0xED

- 0xDD

- 0xCD... etc.