Baofeng UV-5RM Pro GPS - Memory Map & CHIRP Driver Reference

I. General Protocol & Handshake The radio communicates over a serial connection using a random 8-bit XOR cipher for both read and write payloads.

- Baud Rate: Defaults to 19200, but will drop connection and switch to 115200 if the initial handshake fails.

- Handshake Wakeup: PROGRAM + [XOR Seed].

- Handshake Init: INFORMATION + [XOR Seed] (Returns 16 bytes containing the radio identifier).

- Block Size: Reads and writes are executed in 0x1000 (4096) byte chunks.

- Addresses: 0x000000, 0x100000, 0x200000, 0x300000, 0x400000, 0x500000, 0x600000, 0x700000, 0x800000, 0x900000, 0xa00000.

II. Core Memory Regions The EEPROM is partitioned into several distinct blocks.

- 0x0000 - 0x000F: Radio Name (16 bytes, ASCII/GB2312).

- 0x0050 - 0x007F: Device Info (Firmware versions, last programming date).

- 0x0080 - 0x798D: Channel Memory (640 channels, 48 bytes each).

- 0x798E - 0x79A6: Global Settings (Timers, limits, and UI bitmasks).

- 0x79B0 - 0x79B3: Programmable Keys.

- 0x79C0 - 0x79C7: Power-On Password (8 bytes, ASCII).

- 0x7A20 - 0x7A6F: Channel Empty Bitfield (640 bits).

- 0x7B18 - 0x80FF: Zones (10 zones, 152 bytes each).

- 0x8180 - 0x818B: Scan Menu Settings.

- 0x81A0 - 0x81EF: Scan Skip Bitfield (640 bits).

- 0x8200 - 0x83FF: DTMF Settings & PTT IDs.

- 0xA010 - 0xA50F: Contacts (80 contacts, 16 bytes each).

III. Channel Memory Struct (48 Bytes) Each channel starting at 0x0080 consists of the following 48-byte structure:

- 0x00 - 0x03: RX Frequency (Little-Endian BCD).

- 0x04 - 0x07: TX Frequency (Little-Endian BCD).

- 0x08 - 0x09: RX Tone (Big-Endian BCD).

- 0x0A - 0x0B: TX Tone (Big-Endian BCD).

- 0x0C - 0x0F: Unknown/Padding.

- 0x10: Bitfield containing Power (2 bits), Bandwidth (1 bit), Invert (1 bit), Talkaround (1 bit).

- 0x11: Bitfield containing PTT ID Mode (4 bits) and Squelch Mode (4 bits).

- 0x12: Bitfield containing Signaling (4 bits) and Frequency Jump Mode (4 bits).

- 0x13: Bitfield containing Busy Lockout/Launch Banned (1 bit).

- 0x14 - 0x1F: Assorted signaling assignments and padding.

- 0x20 - 0x2F: Channel Name (16 bytes, ASCII/GB2312).

Channel Setting Mappings

| Setting | Decimal Values | String Mapping |
| --- | --- | --- |
| Power | 0, 1, 2 | Low, Medium, High |
| Bandwidth | 0, 1 | NFM, FM |
| PTT ID Mode | 0, 1, 2, 3, 4, 8, 12 | Off, PTT BOT, PTT EOT, PTT Both, 5-Tone BOT, 5-Tone EOT, 5-Tone Both |
| Squelch Mode | 0, 1, 2, 3 | Off, CTDCS, Optional, Both |
| Signaling | 0, 2, 4, 6, 8, 10, 12 | Off, DTMF, 2-Tone, 5-Tone, MDC, BDC1200, BIIS |
| Jump Mode | 0, 1, 2 | Off, Normal, Strict |

IV. Global Settings Bitmasks

Global features are tightly packed into bytes between 0x798E and 0x79A6.

| Offset | Type | Bitmask / Logic | Setting |
| --- | --- | --- | --- |
| 0x798E | Integer | 0-9 | VOX Level |
| 0x7993 | Integer | 0-5 | Auto Power Off (APO) |
| 0x7994 | Integer | 0-14 | Time-Out Timer (TOT) |
| 0x7995 | Integer | 0-10 | Timer Pre-Alarm (TOA) |
| 0x7997 | Integer | val - 12 | GNSS Timezone |
| 0x7999 | Integer | 0-3 | Alert Tone Frequency |
| 0x799D | Integer | 0-9 | NOAA Weather Channel |
| 0x799E | Boolean | 0=Off, 1=On | Power-on Password Enabled |
| 0x79A0 | Bitmask | val & 0x80 | VOX Enabled |
| 0x79A0 | Bitmask | val & 0x40 | GNSS APRS Enabled |
| 0x79A0 | Bitmask | (val & 0x0C) >> 2 | Voice Prompt (0=Off, 1=Chinese, 2=English) |
| 0x79A1 | Bitmask | val & 0x40 | Auto Keypad Lock |
| 0x79A2 | Bitmask | val & 0x80 | Keypad Beep |
| 0x79A2 | Bitmask | (val & 0x60) >> 5 | Roger Beep (0=Off, 1=R1, 2=R2, 3=R3) |
| 0x79A3 | Bitmask | val & 0x80 | GNSS Enabled |
| 0x79A3 | Bitmask | val & 0x04 | Radio Interrupt |
| 0x79A5 | Bitmask | val & 0x03 | Power On Display (0=Pic, 1=Msg, 2=Volt) |
| 0x79A5 | Bitmask | (val & 0x04) >> 2 | Language (0=English, 1=Chinese) |
| 0x79A5 | Bitmask | val & 0x08 | NOAA WX Alert |
| 0x79A6 | Bitmask | (val & 0xE0) >> 5 | Squelch Tail |
| 0x79A6 | Bitmask | val & 0x10 | NOAA Weather Enable |
| 0x79A6 | Bitmask | (val & 0x08) >> 3 | Display Direction (0=Stan, 1=Fail) |

V. Programmable Keys Programmable keys live at 0x79B0 (PF1 Short), 0x79B1 (PF2 Short), 0x79B2 (PF1 Long), and 0x79B3 (PF2 Long).

| Hex Value | Assigned Function |
| --- | --- |
| 0x00 | None |
| 0x01 | SCAN |
| 0x02 | MONITOR |
| 0x04 | FM Radio |
| 0x05 | SOS |
| 0x06 | GNSS System |
| 0x07 | One Touch Search |
| 0x09 | 1750Hz |
| 0x0A | Falling Alarm |
| 0x0B | One Touch Call |
| 0x0C | Zone |
| 0x0D | Battery Display |
| 0x0E | Power |
| 0x0F | Vox |

VI. Identities & Scan Menus

- DTMF ID: 0x822D. Max 3 digits. Stored as raw hex integers, not ASCII.

- 5Tone ID: 0x9443. Max 5 digits. Stored as raw hex integers.

- Scan Mode: 0x8180 (0=TO, 1=CO, 2=SE).

- Scan Memory: 0x818B (0=ENCODER, 1=DECODER, 2=ALL).

VII. Zones & Contacts Layout

- Zones: There are exactly 10 zones. Each zone is exactly 152 bytes (0x98).

  - Offset 0x00: Zone Name (16 bytes).

  - Offset 0x10: Active Channel Count (u16).

  - Offset 0x12: Channel Array (64 u16 integers pointing to channel memory indices).

  - Offset 0x92: Block padding (6 bytes).

- Contacts: There are exactly 80 Contacts. Each contact is exactly 16 bytes.

  - Offset 0x00: Code (u8).

  - Offset 0x01: Unused padding (u8).

  - Offset 0x02: Contact Name (14 bytes).

VIII. Developer "Gotchas"

If rewriting this driver for CHIRP, observe these specific quirks:

- Skipped Hex Values: Baofeng firmware heavily reuses code, meaning many UI menus skip values (e.g., Signaling jumps from 8 to 10 to 12; Prog Keys skip 0x03 and 0x08). Always use CHIRP's RadioSettingValueMap to bind explicit integers instead of RadioSettingValueList to prevent fatal out-of-bounds UI crashes.

- Raw Integer Strings: When parsing DTMF and 5Tone identifiers, do not use Python's chr() function to cast the byte to text. The radio stores 9 as 0x09, which chr() turns into an unprintable control character. Use str(int(byte)) when pulling from memory, and int(char) when packing back to EEPROM.

- Bitwise Safe Operations: Avoid using Python's bitwise NOT operator (~) when masking bits (e.g., val & ~0x40). Due to how CHIRP passes integer casting, this can cause signed integer crashes. Always use explicit hexadecimal masking (e.g., val & 0xBF) when disabling a bit.